LAB SESSION 06

Objective: - To analyze the phenomenon of drift of charge carriers under the influence of electric field.

TASK 1

Apply potential across an intrinsic semiconductor

Q.1: - From the simulation results determine doping, electron and hole density at equilibrium. Don’t forget to attach simulation results.

Q.2: - What is the current density when 0.18V is applied across the semiconductor. Verify the simulation results by mathematical calculation.

MATHEMATICAL CALCULATION:

Jdrift = (Un+Up)niqE

Jdrift = (1350+480)*( 1010)*(1.6x10-19)*(0.18 / 13x10-4)

Jdrift = 4.05x10-4 A/cm2

The simulation results and mathematical calculation result both are same.

Task No.02: - Apply potential across an n-type semiconductor:

Q.1: - From the simulation results determine doping, electron and hole density at equilibrium. Don’t forget to attach simulation results.

Q.2: - What is the current density when 0.18V is applied across the semiconductor. Verify the simulation results by mathematical calculation.

MATHEMATICAL CALCULATION:

Jdrift = UnniqE

Jdrift = (750)*( 1010)*(1.6x10-19)*(0.18 / 13x10-4)

Jdrift = 1.66x10-4 A/cm2

The simulation results and mathematical calculation result both are same.

Task No.03: - Apply a potential across n-type semiconductor. Find out the current density. Now vary the doping and from the simulation results, observe the effect of doping on current density.

Task No.04: - Observe the effect of current saturation at high electric field in n-type semiconductor.

Task No.05: - Observe the effect of current saturation at high electric field in an intrinsic semiconductor.